Topographic Map Downloader

Topo-Map

Content

1 Description	2

2 Introduction	2

3 Usage of the tool	3

3.1 Installation	3

3.2 Usage	3

3.3 Output	4

4 Technical details	5

4.1 Switzerland	5

4.1.1 Downloading 	5

4.1.2 Georeferencing 	5

4.2 France	7

4.2.1 Downloading	7

4.2.2 Calculating Codes	9

4.2.3 Georeferencing	9

4.3 Spain	11

4.3.1 Downloading	11

4.3.2 Georeferencing	11

# Description

TopoMap is a tool to download topographical maps in the scale 1 : 25 000 or 1 : 50 000 of Switzerland , France and Spain. The topographical maps are downloaded from map servers provided by the public Geographic Institutes “Swisstopo”, “Institut Géographique National de France (IGN)” and “Instituto Geografico Militare de España (IGM)”.

# Introduction

Downloading detailed topographic maps in the scale of 1 : 25 000 or 1 : 50 000 of France, Switzerland and Spain, is it really possible? Of course we don’t talk about street maps from Google or Open Street Maps. We talk about professional topographic maps with contour lines, hill shades, forest limits and detailed hiking paths and streets.

Since the public Geographic Institutes of some European Countries started publishing their map products on the Internet, it is possible to view the maps. In the following list you find some popular links:

France:

http://www.geoportail.fr

Spain:

http://www.ign.es/iberpix/visoriberpix/visorign.html

Switzerland:

http://www.mapplus.ch

Germany:

http://www.outdooractive.com/de/touren/

Austria:

http://www.bergfex.at

Unfortunately these links offer only a very limited range of features. The map window is small and it is not possible to visualize own vector data (like GPS waypoints). First and foremost its not possible to download those maps.

Wouldn't it be nice to download the maps? You could view the maps off-line, visualize bigger ranges of the maps, print the maps, copy the maps to your smartphone or process the maps with other tools like Geographical Information Systems (GIS) or Graphic Programs. Unfortunately the sites don't offer a direct download, but TopoMap is able to download them.

Keep in mind, TopoMap doesn’t provide the maps. The maps (respectively the map tiles) are on the Internet. Click on the following link, play around with the numbers i and j and understand, that for instance the entire spain mainland is available:

http://ts4.iberpix.ign.es/tileserver/n=mapa_mtn25;z=30;r=2000;i=1412;j=9149.jpg

So TopoMap doesn’t provide the maps, TopoMap only downloads the tiles, merges them to images (jpg) of a definable size and georeferences them. Therefore TopoMap doesn’t harm copy rights. It’s a tool that helps you saving a lot of time. Imagine you would manually click you through 4 000 000 tiles and merge them with a Graphic Program.

# Usage of the tool

## Installation

Installation couldn’t be easier:

- Download the zip file TopoMap_Release_Executables_X.X.X.X.zip from Source Forge:

http://sourceforge.net/projects/topo-map/

- Unzip all contained files to a new folder. No further installation is needed.

- Make sure, you have the installed the “.NET Framework 3.5”

- Start the executable TopoMap.exe.

## Usage

Select a map that you want to download, change your proxy settings (if needed) and click on start. The program will perform the three mentioned steps:

1. Download of the tiles:

Be aware, that for a country like France, Switzerland or Spain, millions of tiles must be downloaded. The download needs some time (up to weeks) and a lot of hard disk space (up to 50 GBytes).

2. Merging of the tiles to big maps:

One tile corresponds some 100 meters. To create maps, the tiles have to be stitched (merged) together. In the tool a merging size (in meters) can be defined. Depending on your memory size, there is an upper limit for the maximal map image size.

3. Georeferencing the maps:

In the last step the tool creates a World File and an Info File for each map. These files contain the geo-reference information of the map. Basically the files contain the information about the geographic location of the map. A Geographical Information System (GIS) is able to read this information and place the map on the world globe.

All mentioned steps can be interrupted at any time without any loss of information. After a new start, the tool continues the work, where it last was stopped.

Required resources:

|  | Disk Space for download | Disk space for merged maps | Time to download |
| --- | --- | --- | --- |
| France Mainland | 39.9 GBytes | 20.6 GBytes | Ca. 3 weeks |
| France Corsica | 0.8 GBytes | 0.4 GBytes | Ca. 2 days |
| Central Spain | 35.3 GBytes | 20.3 GBytes | Ca. 3 weeks |
| Canariens | 1.1 GBytes | 0.8 GBytes | Ca. 2 days |
| Switzerland | 5.4 GBytes | 3.3 GBytes | Ca. 4 days |

Be aware, that TopoMap downloads a map always in a rectangle. As countries France, Spain or Switzerland have an irregular form, the first hours or days only empty tiles are downloaded. Its recommended to start downloading with a small area like the Canariens or Corsica.

## Output

On the main panel you can define a “Storage Location on the Disk”. The resulting download tiles and the merged images are located under this Storage Location Folder.

Under this folder you find a level directory structure that follows the following sequence logic:

- Name of the nation (for example TOPO25_IB for Spain)

- A Zoom level (not applied for all maps)

- A region of the nation (or example Canariens)

- A tile or map size in meters per meters

- The name of the Row

In the row you find the tiles or maps. The file name applies a x-y coordinate scheme. Rectangle0197632_3069952.jpg for example means the Rectangle (map or tile) at x=0197632 and y=3069952.

Be careful with these coordinates. Depending on the country the coordinates means something different:

- Spain: UTM coordinate in m

- France: Pseudo coordinate system

- Switzerland: Pseudo coordinate system

If you want to know where exactly the tile is located on the globe, have a look into the World-File (*.jgw), the Ozi-Explorer-Calibration-File (*.map) or the info file (*.txt). If you want to learn about World-Files see:

http://en.wikipedia.org/wiki/World_file

# Technical details

How does it work? If you analyze the IP traffic with Wireshark during the navigation on the map web sites, you immediately understand that the Java-Scripts get the maps by requesting small tiles of 256 * 256 pixels (for example in case of France). Well, that's all. You only have to understand the logic behind the HTTP requests, download the tiles sequentially for the whole country, merge them to maps and georeference them. Sounds easy, but depending of the country there is quite a lot of coding work to be done, like session maintaining and map projection calculations.

## Switzerland

### Downloading

TopoMap uses the site http://www.mapplus.ch to download the topographic map of Switzerland. Mapplus stores the tiles in a so called “Google Projection”. Different zoom levels are available. The highest zoom level (16) shows the Swisstopo 1 : 25 000 topographic map.

The URL to a specific tile has the following format. The tile size is 611.5 m * 611.5 m respectively 256 * 256 Pixel.

http://mpa1.mapplus.ch/swisstopo/zoom/X/Y.jpg

Where:

zoom: That's the zoom level. 16 is the highest zoom level

X: Google tile sequence number in x-direction

Y: Google tile sequence number in y-direction

### Georeferencing

From the Google tile sequence numbers and the zoom level it is possible to calculate the geographic location of the upper left corner of the tile:

Longitude(radians) = PI * (2 * (X / 2 ^zoom) – 1)

Latitude (radians) = atan(sinh(– PI * (2 * Y / (2 ^ zoom) – 1 )))

Furthermore it is possible to calculate the coordinates (in meters) of the upper left corner of the tile:

x = + r * PI (2* (X /2^zoom) – 1)

y = – r * PI (2* (Y /2^zoom) – 1)

Where:

r = 6 378 137 m (Radius of the earth)

Note, that the Google Projection is based on the WGS84 Ellipsoid. Common GIS use a so called PRO4 definition to describe the Google Projection. In technical terms it's a “Spherical Mercator Projection”:

"+proj=merc +a=6378137 +b=6378137 +lat_ts=0.0 +lon_0=0.0 +x_0=0.0 +y_0=0 +k=1.0 +units=m +nadgrids=@null +no_defs"

## France

### Downloading

TopoMap uses the site www.goeoportail.fr to download the topographic map of France. The site offers a s set of different layers. TopoMap focuses on the map layer “cartes” and on the zoom levels 1 : 8'000 to 1 : 8'192'000. The zoom level 1 : 8'000 corresponds to the best resolution of the IGN 1 : 25 000 topographic map.

The following link will download a specific tile of France Mainland. The tile size is 512 * 512 m respectively 256 * 256 Pixel.

http://visu-2d.geoportail.fr/geoweb/maps8u6T23uqPyC.jpg

The name of the jpg looks quite much encoded and indeed it is. It contains the following information:

- The set of chosen layers

- The zoom level

- x and y coordinates of the tile in a specific projection

Some reverse engineering of the Java-Scripts helps us understanding the format of the jpg names:

8u6Z (X)XXxPYy.jpg

where:

- 8u6: The selected combination of map layers (8u6 is for the best resolution of the IGN 1 : 25 000 map)

- Z: A letter, that indicates the zoom level

- (X)XXx: Code of the x-coordinate. Details will follow.

- Yy: Code of the y-coordinate. Details will follow.

In the next table you see all the code formats for all different zoom levels:

| Zoom | Geopotrail-Scale | Map-Scale | Code Template | Length | Pixel-Size (m) |
| --- | --- | --- | --- | --- | --- |
| 11 | 8 000 | 25 000 | 8u6T (X)XXx YYy | 10 or 11 | 2 |
| 10 | 16 000 | 25 000 | 8u6Q (X)XXx YYy | 10 or 11 | 4 |
| 9 | 32 000 | 50 000 | 8u6m (X)XXx Yy | 9 or 10 | 8 |
| 8 | 64 000 | 100 000 | 8u6Y (X)XXx Yy | 9 or 10 | 16 |
| 7 | 128 000 | 200 000 | 8u6I (X)XXx Yy | 9 or 10 | 32 |
| 6 | 256 000 |  | 8u6c (X)XXx Yy | 9 or 10 | 64 |
| 5 | 512 000 |  | 8u6j (X)XXx Yy | 9 or 10 | 128 |
| 4 | 1 024 000 |  | 8u6K (X)XXx Yy | 9 or 10 | 256 |
| 3 | 2 048 000 |  | 8u6M (X)XXx y | 8 or 9 | 512 |
| 2 | 4 096 000 |  | 8u67 (X)XXx y | 8 or 9 | 1024 |
| 1 | 8 192 000 |  | 8u6W (X)XXx y | 8 or 9 | 2028 |

Lets now have a closer look at the codes: The codes contain the following 62 characters: 0-9, a-z, A-Z. Some reverse engineering of the Java-Script code of the site is required to understand the order of these characters:

Codes: XuP4NGZ8ngIcjKM7WQTb2qC1ehOotLH9zsmawJSYlAifUvyrkEDx365Fp0VRdB

Nr 10: 00000000001111111111222222222233333333334444444444555555555566

Nr  1: 01234567890123456789012345678901234567890123456789012345678901

Unfortunately there are some nasty irregularities close to Greenwich 0 and close to the equator. First let’s analyze 9 tiles somewhere placed east of Greenwich and north of the equator:

|  | Direction West to East |  |  |
| --- | --- | --- | --- |
| Direction North to South | 8u6T 23uq P y9 | 8u6T 23uC P y9 | 8u6T 23u1 P y9 |
|  | 8u6T 23uq P yz | 8u6T 23uC P yz | 8u6T 23u1 P yz |
|  | 8u6T 23uq P ys | 8u6T 23uC P ys | 8u6T 23u1 P ys |

(X)XX and Y design a block of 62 * 62 tiles. x and y designs the tile within the block.

Now let’s have a look at a first irregularity: The code sequence. The sequence depends on the direction we are talking about. Horizontal (west to east) or vertical (north to south). And it depends where on the globe we are.

|  | West of Greenwich 0 |  | East of Greenwich 0 |  |
| --- | --- | --- | --- | --- |
|  | Horizontal | Vertical | Horizontal | Vertical |
| North of equator | B…X | X…B | X…B | X…B |
| East of equator | B…X | B…X | X…B | B…X |

Te sequence applies to the block codes (blue) and to the tile sequence within the block (red).

There are more irregularities in the horizontal direction in areas close to Greenwich 0:

- The first block east of Greenwich 0 has a block code with only two digits: 2G. The block has 62 tiles.

- The first block west of Greenwich 0 has a block code with only two digits: 9G. The block has one tile less then other blocks, 61 tiles. They start from B to u (not X). There is a simple reason for this irregularity:  Mathematically it exists only one zero, there is not such thing like  positive zero and a negative zero.

- All other blocks are regular, have three digits and 62 tiles.

|  | Horizontal direction (west to east) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Block Codes | 93P | 93u | 9G | 2G | 23u | 23P |
| Tile codes | B…X | B…X | B…u | X…B | X…B | X…B |
| Number of tiles in block | 62 | 62 | 61 | 62 | 62 | 62 |

May be close to the equator there are other irregularities, but they are not relevant in the case of France.

### Calculating Codes

Let us calculate now an example. We have the coordinates of the lower left corner of a specific tile (zoom 1 : 8'000) in the Equirectangular Projection (about the Equirectangular Projection see the next chapter) and we search the download code for the tile.

x=3584 m

y=4974080 m

These coordinates are in meters (m). The zero point is where the Equator and the Greenwich  meet each other. From the table in chapter we know, that the pixel size for the zoom level 1 : 8'000 is 2 m per pixel. All tiles of Geoportail always have 256 * 256 pixel. Therefor the size in meters of our tile is 512 * 512 m.

If we divide the coordinates by 512 m, the result is a tile count value in x- and y-direction:

TileCountX =  3584 m / 512 m = 7

TileCountY = 4974080 m / 512 m = 9715

So our tile is the 7th tile east of  Greenwich and the 9715th tile north of the equator. Now we only have to figure out how to count with the letter codes.

| Positive Tile Counting Numbers |  |  | Negative Tile Counting Numbers |  |  |
| --- | --- | --- | --- | --- | --- |
| Nr | Greenwich to East | Equator to North | Nr | Greenwich to West | Equator to South |
| 0 | 2GX | X | - | - | - |
| 1 | 2Gu | u | -1 | 9Gu | ? |
| 2 | 2GP | P | -2 | 9GP | ? |
| ... |  |  | ... |  |  |
| 7 | 2G8 | 8 | -8 | 9G8 | ? |
| ... |  |  | ... |  |  |
| 61 | 2GB | B | -61 | 9GB | ? |
| 62 | 23uX | XX | -62 | 93uX | ? |
| 63 | 23uu | Xu | -63 | 93uu | ? |
| 64 | 23uP | XP | -64 | 93uP | ? |
| ... |  |  | ... |  |  |
| 123 | 23PB | XB | -123 | 93uB | ? |
| 124 | 23PX | uX | -124 | 93PX | ? |
| 125 | 23Pu | uu | -125 | 93Pu | ? |
| ... |  |  | ... |  |  |
| 9715 | ? | Pzf | -9715 | ? | ? |
| ... |  |  | ... |  |  |

The resulting tile code would be: 8u6T 2G8 Pzf. The corresponding download link would be:

http://visu-2d.geoportail.fr/geoweb/maps8u6T2G8Pzf.jpg

### Georeferencing

Geoportail uses a Coordinate Reference System (CRS) called “Equirectangular Projection”. The 0-Point is where the Greenwich 0 and the equator meet each other. If each tile is 512 * 512 m, we can imagine a coordinate system over the whole world. The x and y coordinate can be directly derived from the (X)XxPYy code.

In the next step it’s possible to calculate the geographic location of any specific x/y coordinate expressed in the “Equirectangular Projection”.

Latitude(degree) = y * 180 / PI / r

Longitude(degree) = x * 180 / PI / r / COS(46.5)

Where:

- r =   6378137 m (Radius of the Earth)

The “Equirectangular Projection” makes sure, distances are correct at a specific latitude. In the case of Mainland France, that's’ at a latitude of 46.5 degrees, more or less in the center of France.

Note, that the Geoportail.fr is based on the WGS84 Ellipsoid. Common GIS use a so called PRO4 definition to describe the Equirectangular Projection:

"+title=Geoportail - France metropolitaine +proj=eqc +nadgrids=null +towgs84=0.0000,0.0000,0.0000 +a=6378137.0000 +rf=298.2572221010000 +lat_0=0.000000000 +lon_0=0.000000000 +lat_ts=46.500000000 +x_0=0.000 +y_0=0.000 +units=m +no_defs"

## Spain

### Downloading

TopoMap uses the site http://www.ign.es/iberpix/visoriberpix/visorign.html to download the topographic map of Spain. Iberpix provides the tiles in UTM projection. The highest zoom level shows the IGM 1 : 25 000 topographic map of Spain.

The following link will download a specific tile of Spain. The tile size is 512 * 512 m respectively 256 * 256 Pixel.

http://ts4.iberpix.ign.es/tileserver/n=mapa_mtn25;z=30;r=2000;i=1212;j=9206.jpg

where

- mtn: The scale of the map (25 means 1 : 25 000)

- z: A UTM zone

- r: A pixel size (2000 means one pixel is 2000 mm in size)

- i, j: The coordinate of the tile in a coordinate system

### Georeferencing

Georeferencing is easy as the tile coordinate (i, j) reflects indirectly the UTM coordinate:

X = i  * 256 Pixel * r / 1000

Y = j * 256 Pixel * r / 1000

Where:

- r: A pixel size (2000 means one pixel is 2000 mm in size)

- i, j: The coordinate of the tile in a coordinate system

Note, that Iberpix maps are based on the WGS84 Ellipsoid. Common GIS use a so called PRO4 definition to describe the UTM projection:

"+proj=utm +zone=30 +ellps=WGS84 +datum=WGS84 +units=m +no_defs"

Depending on the map you want to import in your GIS you have to adapt the zone.

2*62*62 + 32 * 62 + 43 = 9715, 2=P, 32=z , 43=f